Table of Contents

Setup and System Commands	4

SIZE	4

GAP	5

BLINE	6

OFFSET	8

SPEED	9

DENSITY	9

DIRECTION and Mirror Image	10

REFERENCE	10

CODEPAGE	11

CLS	12

FEED	12

BACKFEED	13

FORMFEED	13

HOME	14

PRINT	14

SOUND	15

LIMITFEED	16

SELFTEST	17

Label Formatting Commands	17

BAR	17

BARCODE	18

BITMAP	22

BOX	23

ERASE	24

PUTBMP	25

PUTPCX	26

QRCODE	27

REVERSE	30

TEXT	30

Status Polling Commands(RS-232)	32

<ESC>!?	32

<ESC>!R	33

~!@	33

~!A	33

~!C	34

~!D	34

~!F	34

~!I	35

~!T	35

FileManagement Commands	35

DOWNLOAD	35

EOP	38

FILES	39

KILL	39

MOVE	40

RUN	40

BASIC Commands and Functions	41

ABS( )	41

ASC( )	41

CHR$( )	42

END	42

EOF( )	43

OPEN	44

READ	45

SEEK	46

LOF( )	47

FREAD$( )	47

FOR…NEXT	48

IF…THEN…ELSE…ENDIF	49

GOSUB…RETURN	52

GOTO	53

REM	54

INT( )	54

LEFT$( )	55

LEN( )	55

MID$( )	56

RIGHT$( )	57

STR$( )	57

VAL( )	58

BEEP	59

Device Reconfiguration Commands	59

SET COUNTER	59

SET KEY1, SET KEY2	60

SET PEEL	61

SET TEAR &SET STRIPPER	62

SET HEAD	63

SET RIBBON	63

SET COM1	64

SETPRINTKEY	65

SET REPRINT	66

PEEL	67

KEY1, KEY2	68

Printer Global Variables	68

@LABEL	68

Setup and System Commands

SIZE

[Description]	This command defines the label width and length.

[Syntax]		(1)English system (inch)

SIZE m, n

(2)Metric system (mm)

SIZE m mm, n mm

(3)Dot measurement

This command has been supported since V6.27 EZ and later firmware.

SIZE m dot,n dot

| Parameter | Description |
| --- | --- |
| m | Label width (inch or mm) |
| n | Label length (inch or mm) |

Notes:  203 DPI: 1 mm = 8 dots

300 DPI : 1mm = 12 dots

For metric and dot systems, there must be a space between parameter and “mm” or “dot”.

[Example]		(1) English system (inch)

SIZE 3.5, 3.00

(2) Metric system (mm)

SIZE 100 mm, 100 mm

LABEL

m

GAP

n

m:Label Width

n: Label Height

LABEL

m

GAP

n

m:Label Width

n: Label Height

[See Also]		GAP,BLINE

GAP

[Description]   	Define the gap distance between two labels.

[Syntax]		(1)English system (inch)

GAP m, n

(2)Metric system (mm)

GAP m mm, n mm

(3)Dot measurement

This command has been supported since V6.27 EZ and later firmware.

GAP m dot,n dot

m

m

| Parameter | Description |
| --- | --- |
| m | The gap distance between two labels 0 m 1(inch), 0m 25.4 (mm) 0 ≤ m ≤5 (inch), 0 ≤ m ≤ 127 (mm) / since V6.21 EZ and later firmware |
| n | The offset distance of the gap n  label length (inch or mm) |
| 0,0 | Continuous label |

Feed Direction

Sensor poit

n

m

Feed Direction

Sensor poit

n

m

Notes: 200 DPI : 1 mm = 8 dots

300 DPI : 1mm = 12 dots

For metric system, there must be a space between parameter and mm.

When the sensor type is changed from “Black Mark” to “GAP”, please send the “GAP” command to the printer first.

[Example]		Normal gap

(1).English system (inch)

GAP 0.12,0

(2).Metric system (mm)

GAP 3 mm,0

(3).Continuous label

GAP 0,0

Special gap

(1).English system (inch)

GAP 0.30,0.10

(2).Metric system (mm)

GAP 7.62 mm, 2.54 mm

[See Also]    	SIZE,BLINE

BLINE

[Description]	This command sets the height of the black line and the user-defined extra label feeding length each form feed takes.

[Syntax]		(1)English system (inch)

BLINE m,n

(2)Metric system (mm)

BLINE m mm,n mn

(3) Dot measurement

This command has been supported since V6.27 EZ and later firmware.

BLINE m dot,n dot

| Parameter | Description |
| --- | --- |
| m | The height of black line either in inch or mm 0 ≤ m ≤ 1 (inch), 0 ≤ m ≤ 25.4 (mm) 0 ≤ m ≤5 (inch), 0 ≤ m ≤ 127 (mm) / since V6.21 EZ and later firmware |
| n | The extra label feeding length 0 ≤ n ≤ label length |
| 0,0 | Continuous label |

Note: For metric system, there must be a space between parameter and mm.

    When the sensor type is changed from “GAP” to “Black Mark”, please send the “BLINE” command to the printer first.

     200 DPI : 1 mm = 8 dots

300 DPI : 1mm = 12 dots.

[Example]    	(1) English system (inch)

BLINE 0.20,0.50

(2)Metric system (mm)

BLINE 5.08 mm,12.7 mm

[See Also]		SIZE, GAP

Leading edge

Leading edge

m

m

Black lime

Black lime

Feed Direction

Feed Direction

Leading edge

Leading edge

n

n

Black lime

Black lime

OFFSET

[Description]		This command defines the selective, extra label feeding length 	each form feed takes, which, especially in peel-off mode, is used to 	adjust label stop position, so as for label to register at proper places for 	the intended purposes. The printer back tracks the extra feeding 	length before the next run of printing.

[Syntax]		(1)English system (inch)

OFFSET m

(2)Metric system (mm)

OFFSET m mm

(3)Dot measurement

This command has been supported since V6.27 EZ and later firmware.

OFFSET m dot,n dot

| Parameter | Description |
| --- | --- |
| m | The offset distance (inch or mm) -1 m 1(inch) |

CAUTION: Impropriety offset value may cause paper jam.

For metric system, there must be a space between parameter and mm.

 200 DPI : 1 mm = 8 dots

300 DPI : 1mm = 12 dots

[Example]    	(1) English system (inch)

OFFSET 0.5

(2)Metric system (mm)

FFSET 12.7(mm)

[See Also]		SIZE, GAP, SET PEEL, SET CUTTER

Feed Direction

m

Leading edge

Feed Direction

m

Leading edge

SPEED

[Description]	This command defines the print speed.

[Syntax]		SPEED n

| Parameter | Description |
| --- | --- |
| n | printing speed in inch per second |

[Example]		SPEED 10

[See Also]		DENSITY

DENSITY

[Description]  	This command sets the printing darkness.

[Syntax]		DENSITY n

| Parameter | Description |
| --- | --- |
| n | 0~15 0, specifies the lightest level 15, specifies the darkest level |

Note:

Default DENSITY setting is 8.

[Example]		DENSITY7

DIRECTION and Mirror Image

[Description]	This command defines the printout direction and mirror image. This will be stored in the printer memory.

[Syntax]		DIRECTION n[,m]

| Parameter | Description |
| --- | --- |
| n | 0 or 1. Please refer to the illustrations below: |
| m | 0: Print normal image. 1: Print mirror image. |

Feed Direction

Feed Direction

123456

ABCDEF

TEST PRINT

Feed Direction

Feed Direction

DIRECTION 0，0

DIRECTION 1，0

DIRECTION 0，1

DIRECTION 1，1

Feed Direction

Feed Direction

123456

ABCDEF

TEST PRINT

Feed Direction

Feed Direction

DIRECTION 0，0

DIRECTION 1，0

DIRECTION 0，1

DIRECTION 1，1

[Example]		DIRECTION 0

DIRECTION 0,1

[See Also]		REFERENCE 0,1

REFERENCE

[Description]		This command defines the reference point of the label. The reference 	(origin) point varies with the print direction, as shown:

x

y

y

Direction 1

Direction 0

Feed direction

Reference x,y

Reference x,y

x

x

y

y

Direction 1

Direction 0

Feed direction

Reference x,y

Reference x,y

x

[Syntax]	REFERENCE x, y

| Parameter | Description |
| --- | --- |
| x | Horizontal coordinate (in dots) |
| y | Vertical coordinate (in dots). |

Note:  200 DPI: 1 mm = 8 dots

300 DPI: 1 mm = 12 dots

[Example]		REFERENCE 10,10

[See Also]		DIRECTION

CODEPAGE

[Description]	This command defines the code page of international character set.

[Syntax]		CODEPAGE n

| Parameter | Description |
| --- | --- |
| n | name or number of code page, which can be divided into 8-bit code page further. |

8-bit code page number

437: United States

850: Multilingual

852: Slavic

860: Portuguese

863: Canadian/French

865: Nordic

857: Turkish

Windows code page

1250: Central Europe

1252: LatinI

1253: Greek

1254: Turkish

Note: DATA LENGTH determines 7-bit or 8-bit communications parameter.

[Example]		CODEPAGE 437

[See Also]		COUNTRY, ~!I

CLS

[Description]	This command clears the image buffer.

[Syntax]		CLS

| Parameter | Description |
| --- | --- |
| None | N/A |

Note: This command must be placed after SIZE command.

[Example]   	CLS

[See Also]    	SIZE, GAP, BLINE

FEED

[Description]	This command feeds label with the specified length.

The length is specified by dot.

[Syntax]		FEED n

| Parameter | Description |
| --- | --- |
| n | Unit: dot 1 n  9999 |

[Example]    	FEED 40

Note:  200 DPI: 1 mm = 8 dots

300 DPI: 1 mm = 12 dots

[See Also]		BACKFEED, SIZE, GAP, HOME, FORMFEED

BACKFEED

[Description]		To back feed label with the specified length. The length is specified 	by dot.

[Syntax]		BACKFEED n

TSPL printers only

BACKFEED n

TSPL2 printers only

| Parameter | Description |
| --- | --- |
| n | Unit: dot 1 n  9999 |

[Example]		TSPL printers

BACKFEED 40

TSPL2 printers

BACKFEED 40

CAUTION: Impropriety back feed value may cause paper jam or wrinkle.

Note:  203 DPI: 1 mm = 8 dots

300 DPI: 1 mm = 12 dots

[See Also]		FEED, SIZE, GAP, HOME, FORMFEED

FORMFEED

[Description]	This command feeds label to the beginning of next label.

[Syntax]		FORMFEED

| Parameter | Description |
| --- | --- |
| None | N/A |

[Example]		SIZE 4,2.5

GAP 2 mm,0

DIRECTION 1

FORMFEED

CLS

TEXT 25,25,"3",0,1,1,"FORMFEED COMMAND TEST"

PRINT 1,1

[See Also]		FEED, SIZE, GAP, HOME, BACKFEED

HOME

[Description]		This command will feed label until the internal sensor has determined the origin. Size and gap of the label should be defined before using this command.

[Syntax]     	HOME

| Parameter | Description |
| --- | --- |
| None | N/A |

[Example]		Sample code

SIZE 4,2.5

GAP 2 mm,0

SET COUNTER @0+1

@0="000001"

HOME

CLS

BOX 1,1,360,65,12

TEXT 25,25,"3",0,1,1,"HOME COMMAND TEST"

TEXT 25,80,"3",0,1,1,@0

PRINT 3,1

[See Also]		FEED, SIZE, GAP, FORMFEED

PRINT

[Description]	This command prints the label format stored in the image buffer.

[Syntax]		PRINT m[,n]

| Parameter | Description |
| --- | --- |
| m | Specifies how many sets of labels will be printed. |
|  | 1  m  999999999 |
| n | Specifies how many copies should be printed for each particular label set. |
|  | 1 n 999999999 |

0001

0001

0002

0002

0003

0003

1Set,2Copies

0001

0001

0002

0002

0003

0003

1Set,2Copies

[Example]		SIZE 50 mm,25 mm

GAP 3 mm,0

DIRECTION 1

SET COUNTER @1 1

@1="0001"

CLS

TEXT 10,10,"3",0,1,1,@1

PRINT 3,2

[See Also]		SET COUNTER, DOWNLOAD

SOUND

[Description]		This command is used to control the sound frequency of the beeper. 	There are 10 levels of sounds. The timing control the sound can be set 	by the “interval” parameter.

[Syntax]		SOUND level, interval

| Parameter | Description |
| --- | --- |
| level | Sound level: 0~9 |
| interval | Sound interval: 1~4095 |

[Example]		SOUND 5,200

SOUND 3,200

SOUND 3,200

SOUND 4,200

SOUND 2,200

SOUND 2,200

SOUND 1,200

SOUND 2,200

SOUND 3,200

SOUND 4,200

SOUND 5,200

LIMITFEED

[Description]	If the gap sensor is not set to a suitable sensitivity while feeding labels, the printer will not be able to locate the correct position of the gap. This command stops label feeding and makes the red LED flash if the printer does not locate gap after feeding the length of one label plus one preset value.

[Syntax]		LIMITFEED n (inch, the English system)

LIMITFEED n mm (mm, the metric system)

LIMITFEED n dot(Dot measurement)

| Parameter | Description |
| --- | --- |
| N | The maximum length for sensor detecting |
| Minpaper | The minimum length of paper |
| Maxgap | The maximum length of gap |

[Note]     	The setting will remain resident in memory.

          For metric system, there must be a space between parameter n and mm.

          The default value is 10 inches when printer initializes.

          Since V6.76 EZ, the default value for TDP-225 series printer is 14 inches when printer initializes.

         The setting of parameters “mimpaper” and “maxgap” are using for calibrating the

preprinted label. This parameter has been supported since V6.98.7 EZ..

SELFTEST

[Description]		At this command, the printer will print out the printer information.

[Syntax]		SELFTEST [page]

[Example]		SELFTEST

Label Formatting Commands

BAR

[Description]	This command draws a bar on the label format.

[Syntax]		BAR x, y, width, height

| Parameter | Description |
| --- | --- |
| x | The upper left corner x-coordinate in dot |
| y | The upper left corner y-coordinate in dot |
| width | The width of bar in dot |
| height | The height of bar in dot |

Note: 200 DPI: 1 mm = 8 dots

300 DPI: 1 mm = 12 dots

     Recommended max. bar height is 12 mm at 4” width. Bar height over 12 mm may damage the power supply and affect the print quality.

   Max. print ratio is different for each printer model. Desktop and industrial printer print ratio is limited to 20% and 30% respectively.

[Example]		SIZE 50 mm,25 mm

GAP 3 mm,0

DIRECTION 1

CLS

BAR 80,80,300,100

PRINT 1,1

(100,100)

200

300

(100,100)

200

300

[See Also]		BOX

BARCODE

[Description]	This command is used to print 1D barcodes on label form.

The available bar codes are listed below:

        - Code 128

        - Code 128M

        - EAN 128

        - Interleaved 2 of 5

        - Interleaved 2 of 5 with check digit

        - Code 39 standard

        - Code 39full ASCII

        - Code 39full ASCII with check digit

        - Code 93

        - EAN 13

        - EAN 13 with 2 digits add-on

        - EAN 13 with 5 digits add-on

        - EAN 8

        - EAN 8 with 2 digits add-on

        - EAN 8 with 5 digits add-on

        - Coda bar

        - Post net

        - UPC-A

        - UPC-A with 2 digits add-on

        - UPC-A with 5 digits add-on

        - UPC-E

        - UPC-E with 2 digits add-on

        - UPC-E with 5 digits add-on

        - MSI

        - PLESSEY

        - China POST

        - ITF14

        - EAN14

[Syntax]		BARCODE X,Y,"code type",height,human readable,rotation,narrow,wide,[alignment,]"content ".

| Parameter | Description |
| --- | --- |
| x | Specify the x-coordinate of the bar code on label |
| y | Specify the y-coordinate of the bar code on label |
| Codetype 128 | Code 128, switching code subset A, B, C automatically. |
| 128M | Code 128, switching code subset A, B, C manually |

| Controlcode | A | B | C |
| --- | --- | --- | --- |
| 096 | FNC3 | FNC3 | NONE |
| 097 | FNC2 | FNC2 | NONE |
| 098 | SHIFT | SHIFT | NONE |
| 099 | CODE C | CODE C | NONE |
| 100 | CODE B | FNC4 | CODE B |
| 101 | FNC4 | CODE A | CODE A |
| 102 | FNC1 | FNC1 | FNC1 |
| 103 | Start (CODE A) |  |  |
| 104 | Start (CODE B) |  |  |
| 105 | Start (CODE C) |  |  |

Use “!” as a starting character for the control code followed by three control codes.

If the start subset is not set, the default starting subset is B.

        - EAN128Code 128, switching code subset A, B, C automatically

        - 25Interleaved 2 of5

        - 25C        Interleaved 2 of5 with check digits

        - 39Auto switch full ASCII and standard code 39 for PLUS

models.

        - 39C       Code 39full ASCII with check digit

Code 39 standard with check digit

Auto switch full ASCII and standard code 39 for PLUS models.

        - 39S       Code 39 standard

        - 93        Code 93

        - EAN 13EAN 13

        - EAN 13+2EAN 13 with 2 digits add-on

        - EAN 13+5EAN 13 with 5 digits add-on

        - EAN 8EAN 8

        - EAN 8+2EAN 8 with 2 digits add-on

        - EAN 8+5EAN 8 with 5 digits add-on

        - CODACodabar

        - POST Post net

        - UPCAUPC-A

        - UPCA+2UPC-A with 2 digits add-on

        - UPCA+5UPC-A with 5 digits add-on

        - UPCEUPC-E

        - UPCE+2UPC-E with 2 digits add-on

        - UPCE+5   UPC-E with 5 digits add-on

        - CPOST China post code

        - MSIMSI code

        - MSIC

        - PLESSEYPLESSEY code

        - ITF 14ITF 14 code

        - EAN 14EAN 14 code

        - Height bar code height expressed by dot

        - human readable 0: human not readable

1: human readable

2: human readable aligns to center

3: human readable aligns to right

        - rotation Rotate bar code clockwise in degrees

0       non rotation

90      rotate 90 degrees clockwise

180     rotate 180 degrees clockwise

270rotate 270 degrees clockwise

Narrow

width of narrow element in dot

Wide

width of wide element in dot

|  | narrow:wide 1:1 | narrow: wide 1:2 | narrow: wide 1:3 | narrow: wide 2:5 | narrow: wide 3:7 |
| --- | --- | --- | --- | --- | --- |
| 128 | 10x | N/A | N/A | N/A | N/A |
| EAN128 | 10x | N/A | N/A | N/A | N/A |
| 25 | N/A | 10x | 10x | 5x | N/A |
| 25C | N/A | 10x | 10x | 5x | N/A |
| 39 | N/A | 10x | 10x | 5x | N/A |
| 39C | N/A | 10x | 10x | 5x | N/A |
| 93 | N/A | N/A | 10x | N/A | N/A |
| EAN13 | 8x | N/A | N/A | N/A | N/A |
| EAN13+2 | 8x | N/A | N/A | N/A | N/A |
| EAN13+5 | 8x | N/A | N/A | N/A | N/A |
| EAN8 | 8x | N/A | N/A | N/A | N/A |
| EAN8+2 | 8x | N/A | N/A | N/A | N/A |
| EAN8+5 | 8x | N/A | N/A | N/A | N/A |
| CODA | N/A | 10x | 10x | 5x | N/A |
| POST | 1x | N/A | N/A | N/A | N/A |
| UPCA | 8x | N/A | N/A | N/A | N/A |
| UPCA+2 | 8x | N/A | N/A | N/A | N/A |
| UPCA+5 | 8x | N/A | N/A | N/A | N/A |
| UPCE | 8x | N/A | N/A | N/A | N/A |
| UPCE+2 | 8x | N/A | N/A | N/A | N/A |
| UPCE+5 | 8x | N/A | N/A | N/A | N/A |
| CPOST | N/A | N/A | N/A | N/A | 1x |
| MSI | N/A | N/A | 10x | N/A | N/A |
| MSIC | N/A | N/A | 10x | N/A | N/A |
| PLESSY | N/A | N/A | 10x | N/A | N/A |
| ITF14 | N/A | 10x | 10x | 5x | N/A |
| EAN14 | N/A | N/A | N/A | 5x | N/A |

code number the maxi mum number of digits of bar code content

| Barcode type | Maximum bar Code length | Barcode type | Maximum bar Code length |
| --- | --- | --- | --- |
| 128 | — | POST | 5,9,11 |
| EAN128 | — | UPCA | 11 |
| 25 | — | UPCA+2 | 13 |
| 25C | — | UPCA+5 | 16 |
| 39 | — | UPCE | 6 |
| 39C | — | UPCE+2 | 8 |
| 93 | — | UPCE+5 | 11 |
| EAN13 | 12 | CPOST | — |
| EAN13+2 | 14 | MSI | — |
| EAN13+5 | 17 | MSIC | — |
| EAN8 | 7 | PLESSY | — |
| EAN8+2 | 9 | ITF14 | 13 |
| EAN8+5 | 12 | EAN14 | 13 |
| CODA | — |  |  |

[Example]		BARCODE 100,100,”39”,96,1,0,2,4,”1000”

BARCODE 10,10,”128M”,48,1,0,2,2,”!104!096ABCD!101EFGH”

(The above example of code 128M encoded with CODE B start 	character. The next character will be the code 128function character 	FNC3 which is then followed by the ABCD characters and EFGH 	characters encoded as CODE A subset.

BITMAP

[Description]	This command is used to draw bitmap images (Not BMP graphic file).

[Syntax]      	BITMAP X, Y, width, height, mode, bitmap data…

| Parameter | Description |
| --- | --- |
| x | Specify the x-coordinate of the bitmap image |
| y | Specify the y-coordinate of the bitmap image |
| width | The width of the image in bytes |
| height | The height of the image in dot |
| mode | Graphic mode is listed below: |
| 0 | OVERWRITE |
| 1 | OR |
| 2 | XOR |
| bitmap data | The bitmap data |

| ROW (Y- axis) | L-Byte |  | R-Byte |  |
| --- | --- | --- | --- | --- |
|  | Binary | Hexadecimal | Binary | Hexadecimal |
| 0 | 00000000 | 00 | 00000000 | 00 |
| 1 | 00000000 | 00 | 00000000 | 00 |
| 2 | 00000000 | 00 | 00000000 | 00 |
| 3 | 00000111 | 07 | 11111111 | FF |
| 4 | 00000011 | 03 | 11111111 | FF |
| 5 | 00010001 | 11 | 11111111 | FF |
| 6 | 00011000 | 18 | 11111111 | FF |
| 7 | 00011100 | 1C | 01111111 | 7F |
| 8 | 00011110 | 1E | 00111111 | 3F |
| 9 | 00011111 | 1F | 00011111 | 1F |
| A | 00011111 | 1F | 10001111 | 8F |
| B | 00011111 | 1F | 11000111 | C7 |
| C | 00011111 | 1F | 11100011 | E3 |
| D | 00011111 | 1F | 11110111 | F7 |
| E | 00011111 | 1F | 11111111 | FF |
| F | 00011111 | 1F | 11111111 | FF |

[Example]		SIZE 4,2

GAP 0,0

CLS

BITMAP 200,200,2,16,0,

PRINT 1,1

[See Also]		PUTBMP, PUTPCX

BOX

[Description]	This command is used to draw rectangles on the label.

[Syntax]      	BOX x,y,x_end,y_end,line thickness[,radius]

| Parameter | Description |
| --- | --- |
| X_start | Specify x-coordinate of upper left corner in dot |
| Y_start | Specify y-coordinate of upper left corner in dot |
| X_end | Specify x-coordinate of lower right corner in dot |
| Y_end | Specify y-coordinate of lower right corner in dot |
| line thickness | Line thickness of the box in dot |
| radius | Optional. Specify the round corner. Default is 0. *Since V5.28 EZ |

Note:  200 DPI: 1 mm = 8 dots

300 DPI: 1 mm = 12 dots

     Recommended max. thickness of box is 12 mm at 4” width. Thickness of box larger than 12 mm may damage the power supply and affect the print quality. Max. print ratio is different for each printer model. Desktop and industrial printer print ratio is limited to 20% and 30% respectively.

[Example]		SIZE 4,1.1

CLS

BOX 60,60,610,210,4

BOX 80,80,590,190,4

BOX 100,100,570,170,4,20

BOX 120,120,550,150,4,20

PRINT 1

(100,100)

(200,200)

(100,100)

(200,200)

[See Also]    	BAR

ERASE

[Description]	This command is used to clear a specified region in image buffer.

[Syntax]     	ERASE X_start, Y_start, X_width, Y_height

| Parameter | Description |
| --- | --- |
| X_start | The x-coordinate of the starting point in dot |
| Y_start | The y-coordinate of the starting point in dot |
| X_width | The region width in x-axis direction in dot |
| Y_height | The region height in y-axis direction in dot |

[Example]    	SIZE 4,2.5

GAP 0,0

DIRECTION 1

CLS

BAR 100,100,300,300

ERASE 150,150,200,200

PRINT 1,1

[See Also]		CLS

(100,100)

(150,150)

300

300

200

200

(100,100)

(150,150)

300

300

200

200

PUTBMP

[Description]  	This command prints BMP format images. The grayscale printing is for direct thermal mode only.Support 1-bit (monochrome) and 8-bit (256-color) BMP graphic only.

[Syntax]      	PUTBMP X, Y, “filename "[, bpp][, contract]

| Parameter | Description |
| --- | --- |
| X | The x-coordinate of the BMP format image |
| Y | The y-coordinate of the BMP format image |
| filename | The downloaded BMP filename. |
| bpp | Optional. Bits per pixel of grayscale graphic. Default is 1. *Since V6.91EZ. 1: 1-bit (monochrome) graphic 8: 8-bit (256-color) graphic |
| contrast | Optional. Contrast of grayscale graphic. Default is 80. Suggested range is from 60 to 100. *Since V6.91EZ. |

[Example]

[See Also]		DOWNLOAD, BITMAP, PUTPCX

PUTPCX

[Description]  	This command prints PCX format images. TSPL language supports 2-color PCX format graphics. TSPL2 language supports 256-color PCX format graphics.

[Syntax]      	PUTPCX X, Y, “filename”

| Parameter | Description |
| --- | --- |
| X | The x-coordinate of the PCX image |
| Y | The y-coordinate of the PCX image |
| filename | The downloaded PCX filename. Case sensitive |

[Example]

[See Also]		DOWNLOAD, BITMAP, PUTPCX

QRCODE

[Description]  	This command prints QR code.

[Syntax]      	QRCODE x,y,ECC Level,cell width,mode,rotation,[model,mask,]"content"

| Parameter | Description |
| --- | --- |
| X | The upper left corner x-coordinate of the QR code |
| Y | The upper left corner y-coordinate of the QR code |
| ECC level | Error correction recovery level L : 7% M : 15% Q : 25% H : 30% |
| cell width | 1~10 |
| mode | Auto / manual encode A : Auto M : Manual |
| rotation | 0 : 0 degree 90 : 90 degree 180 : 180 degree 270 : 270 degree |
| model | M1: (default), original version M2: enhanced version (Almost smart phone is supported by this version.) |
| mask | S0~S8, default is S7 |
| content | The encodable character set is described as below, Encodable character set: 1) Numeric data: (digits 0~9) 2) Alphanumeric data Digits 0-9 Upper case letters A-Z Nine other characters: space, $ % * + - . / : ) 3) 8-bit byte data JIS 8-bit character set (Latin and Kana) in accordance with JIS X 0201 4) Kanji characters Shift JIS values 8140 HEX –9FFC HEX and E040 HEX –EAA4 HEX . These are values shifted from those of JIS X 0208. Refer to JIS X 0208 Annex 1 Shift Coded Representation for detail. Data characters per symbol (for maximum symbol size): Model 1 (Version 14-L) Model 2 (Version 40-L) Numeric data 1,167 characters 7,089 characters Alphanumeric data 707 characters 4,296 characters 8-bit byte data 486 characters 2,953 characters Kanji data 299 characters 1,817 characters * If "A" is the first character in the data string, then the following data after "A" is alphanumeric data. *If "N" is the first character in the data string, then the following data after "N" is numeric data. *If "B" is the first character in the data string, then the following 4 digits after "B" is used to specify numbers of data. After the 4 digits is the number of bytes of binary data to be encoded. *If "K" is the first character in the data string, then the following data after "K" is Kanji data. *If "!" is in the data string and follows by "N", "A", "B", "K" then it will be switched to specified encodable character set. Manual mode example: QRCODE 100,10,L,7,M,0,M1,S1,"ATHE FIRMWARE HAS BEEN UPDATED" (Where A: Alphanumeric data) QRCODE 100,10,M,7,M,0,M1,S2,"N123456" (Where N: Numeric data) QRCODE 100,10,Q,7,M,0,M1,S3,"N123456!ATHE FIRMWARE HAS BEEN UPDATED" (Where N: Numeric data ; !:Transfer char ; A: Alphanumeric data) QRCODE 100,10,H,7,M,0,M1,S3,"B0012Product name" (where B: Binary data ; 0012: 12 bytes ) QRCODE 100,10,M,7,M,0,M1,S3,"K" (Where K: Kanji data) Auto mode example: QRCODE 100,10,M,7,A,0,"THE FIRMWARE HAS BEEN UPDATED" |

[Example]    	SIZE 4,2.5

GAP 0,0

DIRECTION 1

CLS

QRCODE 10,10,H,4,A,0,"ABCabc123"

QRCODE 160,160,H,4,A,0,"123ABCabc"

QRCODE 310,310,H,4,A,0," 印表機 ABCabc123"

PRINT 1,1

REVERSE

[Description]  	This command is used to reverse a region in image buffer.

[Syntax]      	REVERSE X_start, Y_start, X_width, Y_height

| Parameter | Description |
| --- | --- |
| X_start | The x-coordinate of the starting point in dot |
| Y_start | The y-coordinate of the starting point in dot |
| X_width | The region width in x-axis direction in dot |
| Y_height | The region height in y-axis direction in dot |

Note:  200 DPI: 1 mm = 8 dots

300 DPI: 1 mm = 12 dots

      Recommended max. height of reversed black area is 12mm at 4” width. Height of reversed area that is larger than 12 mm may damage the power supply and affect the print quality.

      Max. print ratio is different for each printer model. Desktop and industrial printer print ratio is limited to 20% and 30% respectively.

[Example]    	SIZE 4,2.5

GAP 0,0

DIRECTION 1

CLS

TEXT 100,100,"3",0,1,1,"REVERSE"

REVERSE 90,90,128,40

PRINT 1,1

TEXT

[Description]  	This command is used to print text on label.

[Syntax]      	TEXT X, Y, “font”, rotation, x-multiplication, y-multiplication, “content”

| Parameter | Description |
| --- | --- |
| X | The x-coordinate of the text |
| Y | The y-coordinate of the text |
| Font | Font name |
| 0 | Monotye CG Triumvirate Bold Condensed, font width and height is stretchable. |
| 1 | 8 x 12fixed pitch dot font |
| 2 | 12 x 20fixed pitch dot font |
| 3 | 16 x 24 fixed pitch dot font |
| 4 | 24 x 32fixed pitch dot font |
| 5 | 32 x 48 dot fixed pitch font |
| 6 | 14 x 19 dot fixed pitch font OCR-B |
| 7 | 21 x 27 dot fixed pitch font OCR-B |
| 8 | 14 x25 dot fixed pitch font OCR-A |
| TST24.BF2 | Traditional Chinese 24 x 24font |
| TSS24.BF2 | Simplified Chinese 24 x 24font(GB) |
| K | Korean 24 x 24font (KS) |
| Rotation | The rotation angleof text |
| 0 | 0 degree |
| 90 | 90 degrees, in clockwise direction |
| 180 | 180 degrees, in clockwise direction |
| 270 | 270 degrees, in clockwise direction |
| X-multiplication: | Horizontal multiplication, upto10x. Available factors:1~10width(point)of true type font. 1 point=1/72 inch. |
| Y-multiplication: | Vertical multiplication, up to 10x. Available factors: 1~10 For true type font, this parameter is used to specify the height (point) of true type font. 1 point=1/72 inch. |

Note: The internal font (font #1~#5) pitch between TSPL and TSPL2 is different.

     Font "0" and "ROMAN.TTF" internal True Type Fonts are available in TSPL2 language

printers, but not TSPL language printers.

     If there is any double quote (") within the text, please change it to \["].

    If font "0" is used, the font width and font height is stretchable by x-multiplication and

y-multiplication parameter. It is expressed by pt (point). 1 point=1/72inch.

  EPL2 and ZPL2 are emulating for Eltron ® and Zebra ® languages.

[Example]   	SIZE 3,2

GAP 0,0

CLS

TEXT 100,100,”5”,0,1,1,”\[“]DEMO FOR TEXT\[“]”

TEXT 100,200,”0,1,20,”\[“]True Type Font Test Print\[“]”

PRINT 1,1

Status Polling Commands(RS-232)

<ESC>!?

[Description]  	This command obtains the printer status at any time, even in the event of printer error. An inquiry request is solicited by sending an <ESC> (ASCII 27, escape character) as the beginning control character to the printer. A one byte character is returned, flagging the printer status. A 0 signifies the printer is ready to print labels.

| Bit | Status |
| --- | --- |
| 0 | Head opened |
| 1 | Paper jam |
| 2 | Out of paper |
| 3 | Out of ribbon |
| 4 | Pause |
| 5 | Printing |
| 6 | Cover opened (option) |
| 7 | Environment Temperature over range (option) |

| Hex Receive | Printer Status |
| --- | --- |
| 00 | Normal |
| 01 | Head opened |
| 02 | Paper Jam |
| 03 | Paper Jam and head opened |
| 04 | Out of paper |
| 05 | Out of paper and head opened |
| 08 | Out of ribbon |
| 09 | Out of ribbon and head opened |
| 0A | Out of ribbon and paper jam |
| 0B | Out of ribbon, paper jam and head opened |
| 0C | Out of ribbon and out of paper |
| 0D | Out of ribbon, out of paper and head opened |
| 10 | Pause |
| 20 | Printing |

[Syntax]     	<ESC>! ?

[See Also]  	<ESC>! S

<ESC>!R

[Description]  		This command resets the printer. The beginning of the command is an ESCAPE character (ASCII 27).The files downloaded in memory will be deleted. This command cannot be sent in dump mode.

[Syntax]      	<ESC>! R

| Parameter | Description |
| --- | --- |
| N/A | N/A |

[See Also]  	<ESC>! ?

~!@

[Description]  	This command is used to inquire the mileage of the printer. The integer part of mileage is returned (the decimal part of mileage is not return).  It is returned to PC in ASCII characters. The ending character of mileage is 0x0D.

[Syntax]      	~! @

| Parameter | Description |
| --- | --- |
| N/A | N/A |

[Example]  	~! @

~!A

[Description]  	This command is used to inquire about the free memory of the printer. The number of bytes of free memory is returned in decimal digits, with 0x0d as ending code of PC.

[Syntax]      	~!A

| Parameter | Description |
| --- | --- |
| None | N/A |

[Example]  	~!A

[See Also] 		FILES

~!C

[Description]  	This command inquires the presence of Real Time Clock. One byte is return from the printer, indicating whether or not the RTC is installed. This command is only for the firmware before V6.xx.

[Syntax]      	~!C

| Parameter | Description |
| --- | --- |
| None | N/A |

| Return value | Description |
| --- | --- |
| 0 | RTC is not installed. |
| 1 | RTC is installed. |

[Example]  	~!C

[See Also] 		FILES

~!D

[Description]  	This command enters the printer into DUMP mode. In DUMP mode, the printer outputs code directly without interpretation.

[Syntax]      	~!D

| Parameter | Description |
| --- | --- |
| None | N/A |

[Example]  	~!D

~!F

[Description]  	This command inquires all about files resident in the printer memory, and fonts installed in the memory module. The filename are returned in ASCII characters. Each file name ends with 0x0D. The ending character is 0x1A. Entering this command multiple times will cycle through the files resident on memory.

[Syntax]     	~!F

| Parameter | Description |
| --- | --- |
| None | N/A |

[Example]  	~!F

[See Also]  	FILES

~!I

[Description]  	The command inquires the code page and country setting of the printer.

[Syntax]      	~!I

| Parameter | Description |
| --- | --- |
| None | N/A |

[Example]  	~!I

[See Also]  	COUNTRY, CODEPAGE

~!T

[Description]  	This command is used to inquire the model name and number of the printer. They are returned in ASCII characters.

[Syntax]     	~!T

| Parameter | Description |
| --- | --- |
| None | N/A |

[Example]  	~!T

FileManagement Commands

DOWNLOAD

[Description]  "DOWNLOAD" is a header of the file that is to be saved in the printer's memory. The downloaded files can be divided into two categories: program files and data files (including text data files, PCX graphic files and bitmap font files) The detailed descriptions regarding the download syntax for different files are as follows:

Maximum numbers of file saved in DRAM:

50 files for TSPL/TSPL2 language printers

Maximum numbers of file saved in Flash memory:

50 files for TSPL language printers

256 files for TSPL2 language desktop printers (TTP/TDP-245 / TTP-343 / TTP-244)

256 files for TSPL2 language industrial printers (TTP-246M/344M)

If "AUTO.BAS" exists in the printer memory, it will be automatically executed upon printer startup.To disable the auto execution function, please follow the procedures below.

For TTP-245/TTP-343/TDP-245 series, 245C/343C/244CE series

Hold the FEED key and power on the switch. The LED color will be changed as following pattern.

Orange   red (5 blinks)   orange (5 blinks)   green (5 blinks)   solid green (for firmware version before V3.37)

Orange   red (5 blinks)   orange (5 blinks)   green (5 blinks)   green and orange (5 blinks)   red and orange (5 blinks)   solid green (V3.37)

Release the FEED key while LED becomes solid green to prevent the printer from running “AUTO.BAS”.

For TTP-246M/TTP-344M series

Hold the FEED key and power on the switch. The ERROR LED will be on. Printer is now ready to use.

For TTP-2410M/TTP-246M PLUS series

Hold the PAUSE and FEED keys and power on the switch. “AUTO.BAS” will not be executed after printer initialization, and will now be ready for use.

Alternatively, hold the PAUSE key and power on the switch. After sensor calibration, the “AUTO.BAS” will not be executed. Printer is now ready for use.

[Syntax]      	1. Download a program file

DOWNLOAD[n,]“FILENAME.BAS”

| Parameter | Description |
| --- | --- |
| n | Specify the memory which is used to save the download files. |
| n is ignored | Download files to DRAM only. If you would like to save the files from DRAM to Flash memory before turning off power, you can issue MOVE command to printer to move the files from DRAM to Flash memory. F: Download files to main board flash memory. |
| FILENAME.BAS | The filename resident in printer memory. |

Note:

(1).The filename is case sensitive.

(2).The extension of the program file must be “.BAS”

(3). Filename format must be in 8.3 format.

(4). It should use with EOP command.

(5). If memory is not specified, all files will be downloaded to DRAM.

(6). The priority of AUTO.BAS in each memory device:

A. DRAM > FLASH > CARD (Ext. FLASH) if firmware is before V6.80EZ.

B. DRAM > CARD (Ext. FLASH) > FLASH if firmware is after V6.80EZ (include).

(7). No Battery is used to back up files in DRAM. which will be lost in the event printer power is lost.

2. Download a data file

DOWNLOAD[n,]“FILENAME”, DATA SIZE, DATA CONTENT…where

| Parameter | Description |
| --- | --- |
| n | Specify the memory location to save the download files. |
| n is ignored | Download files to DRAM only. If you would like to save the files from DRAM to Flash memory before turning off power, you can issue MOVE command to printer to move the files from DRAM to Flash memory. F: Download files to main board flash memory. |
| FILENAME.BAS | The name of data file that will remain resident in the printer memory. It is case sensitive. |
| DATA SIZE | The actual size (numbers of byte) of the data file without header. |

Note:

(1).For text data file, CR (carriage return) 0x0D and LF (Line Feed) 0x0A is the separator of data.

(2).If memory is not specified, all files will be downloaded to DRAM.

No Battery is used to backup DRAM.

(3).No Battery is used to back up files in DRAM. which will be lost in the event printer power is lost.

(5).When writing a download program, “DOWNLOAD” header must be placed in the beginning of file, and “EOP” must be placed at the end of program.

(6).To run the program, call the main filename without BAS extension or use RUN command to start the download program.

[Example]  	The example program listed below will download to printer SDRAM.

DOWNLOAD "EXAMPLE.BAS"

SIZE 4,4

GAP 0,0

DIRECTION 1

SET TEAR ON

CLS

TEXT 100,100,"3",0,1,1,"EXAMPLE PROGRAM"

PRINT 1

EOP

[See Also]  	EOP, RUN, PUTBMP, PUTPCX, INPUT, FILES, ~!F

EOP

[Description]  	End of program. To declare the start and end of BASIC language commands used in a program, the DOWNLOAD “FILENAME.BAS” must be added in the first line of the program, and “EOP” statement at the last line of program.

[Syntax]       EOP

[Example]  	DOWNLOAD "DEMO.BAS"

SIZE 4,4

GAP 0,0

DIRECTION 1

SET TEAR ON

CLS

TEXT 100,100,"3",0,1,1,"DEMO PROGRAM"

PRINT 1

EOP

[See Also]  	DOWNLOAD, INPUT, FILES, ~!F

FILES

[Description]  	This command prints out the total memory size, available memory size and files lists (or lists the files through RS-232) in the printer memory (both FLASH memory and DRAM).

[Syntax]       FILES

[See Also]  	~!F, KILL

KILL

[Description]  	This command deletes a file in the printer memory. The wild card (*) will delete all files resident in specified DRAM or FLASH memory.

[Syntax]       KILL[n],”FILENAME”

| Parameter | Description |
| --- | --- |
| n | Specify the memory location that files will be deleted. |
| n is ignored | Kill files saved in DRAM. F: Kill files from main board flash memory. E: Kill files from expansion memory module. |
| FILENAME | The name of data file that will delete in the printer memory (case sensitive) |

Note:

(1). If optional parameter n is not specified, firmware will delete the file in DRAM.

Syntax example

1. KILL “FILENAME”

2. KILL “*.PCX”

3. KILL “*”

4. KILL F,“FILENAME”

5. KILL E,“*.PCX”

[Example]  Users can use printer SELFTEST utility to list printer configurations and files saved in the printer memory, or use the FILES command to print the downloaded file list in printer. Follow the steps below to delete files in the printer memory via parallel port connection.

C:\>COPY CON LPT1<ENTER>

FILES<ENTER>

<CTRL><Z><ENTER>

C:\>COPY CON LPT1<ENTER>

KILL “DEMO.BAS” <ENTER>

<CTRL><Z><ENTER>

C:\>COPY CON LPT1<ENTER>

FILES<ENTER>

<CTRL><Z><ENTER>

Note:

<ENTER> stands for PC keyboard “ENTER” key.

<CTRL><Z> means to hold PC keyboard “CTRL” key then press the PC keyboard <Z> key

[See Also]  	~!F, FILES

MOVE

[Description]  	This command is used to move downloaded files from DRAM to the FLASH memory.

[Syntax]       MOVE

| Parameter | Description |
| --- | --- |
| N/A | N/A |

[See Also]  	DOWNLOAD, EOP

RUN

[Description]   This command executes a program resident in the printer memory. It is available for TSPL2 language printers only.

[Syntax]       RUN “FILENAME.BAS”

[Example]     C:\>COPY CON LPT1<ENTER>

RUN “DEMO.BAS”<ENTER>

<CTRL><Z><ENTER>

C:\>

Note: This command can be replaced to filename that without typing “.BAS”.

[See Also]  	DOWNLOAD, EOP

BASIC Commands and Functions

ABS( )

[Description]   This function returns the absolute value of an integer, floating point or variable.

[Syntax]      ABS (VARIABLE)

[Example]     DOWNLOAD "TEST.BAS"

SIZE 4,4

GAP 0,0

DIRECTION 1

SET TEAR ON

CLS

A=ABS(-100)

B=ABS(-50.98)

C=-99.99

TEXT 100,100,"3",0,1,1,STR$(A)

TEXT 100,150,"3",0,1,1,STR$(B)

TEXT 100,200,"3",0,1,1,STR$(ABS(C))

PRINT 1

EOP

RUN "TEST.BAS" [See Also]     DOWNLOAD, EOP

[See Also]  	DOWNLOAD, EOP

ASC( )

[Description]   This function returns the ASCII code of the character.

[Syntax]       ASC(“A”)

[Example]     DOWNLOAD "TEST.BAS"

SIZE 4,4

GAP 0,0

DIRECTION 1

SET TEAR ON

CLS

CODE1=ASC("A")

TEXT 100,100,"3",0,1,1,STR$(CODE1)

PRINT 1

EOP

RUN "TEST.BAS"

[See Also]     DOWNLOAD, EOP, STR$()

CHR$( )

[Description]   This function returns the character that has the specified ASCII code.

[Syntax]       CHR$(n)

| Parameter | Description |
| --- | --- |
| n | The ASCII code |

[Example]     DOWNLOAD "TEST.BAS"

SIZE 4,4

GAP 0,0

DIRECTION 1

SET TEAR ON

CLS

A=75

WORD$=CHR$(A)

TEXT 100,100,"3",0,1,1,WORD$

PRINT 1

EOP

RUN "TEST.BAS"

[See Also]     DOWNLOAD, EOP, STR$(), ASC$()

END

[Description]   This command states the ending of program.

[Syntax]       END

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 4,2

GAP 0,0

DIRECTION 1

CLS

TEXT 200,60,"4",0,1,1,"END COMMAND TEST"

X=300

Y=200

X1=500

Y1=400

GOSUB DR_LINE

PRINT 1

END

:DR_LINE

FOR I=1 TO 100 STEP 10

BOX X+I,Y+I,X1-I,Y1-I,5

NEXT

RETURN

EOP

DEMO

[See Also]     DOWNLOAD, EOP, GOSUB

EOF( )

[Description]   This function is used to detect an opened download file to see whether it has reached the end of file.

[Syntax]       EOF (File Handle)

| Parameter | Description |
| --- | --- |
| File handle | Either 0 or1 |

| Parameter | Description |
| --- | --- |
| None-zero | End of file |
| 0 | Not end of file |

[Example]     DOWNLOAD "DATA",16,COMPUTER

2000

DOWNLOAD "DEMO.BAS"

SIZE 3,3

GAP 0.0,0

DIRECTION 1

CLS

OPEN "DATA",0

SEEK 0,0

Y=110

TEXT 10,10,"3",0,1,1,"*****EOF TEST*****"

:A

Temp$=""

READ 0,ITEM$,P

TEXT 10,Y,"2",0,1,1,ITEM$+"$"+STR$(P)+"[EOF(0)="+STR$(EOF(0))+"]"

BARCODE 10,Y+25,"39",40,1,0,2,4,"PRICE-"+STR$(P)

Y=Y+100

IF EOF(0)=0 THEN GOTO A

PRINT 1

EOP

DEMO

[See Also]     DOWNLOAD, EOP, OPEN, READ, SEEK

OPEN

[Description]   This command opens a downloaded file and establishes the file handle. Up to two file handles are supported, thus only up to two files can be opened simultaneously. The file to be opened should be downloaded prior to using this command. When opening a file, the firmware will search automatically to see if the file exists in the on board flash memory or extended memory card. *Since V6.37 EZ, if the file doesn’t exist, the printer will create this file in the onboard FLASH.

[Syntax]       OPEN “Filename”, File handle

| Parameter | Description |
| --- | --- |
| Filename | The file downloaded in the printer memory. |
| File handle | Either 0 or 1. |

[Example]     DOWNLOAD "DATA.DAT",18,Open file in DRAM.

DOWNLOAD F,"DATA.DAT",19,Open file in FLASH.

DOWNLOAD "TEST.BAS"

data1$=""

data2$=""

data3$=""

OPEN "DATA.DAT",0

READ 0,data1$

CLOSE 0

OPEN F,"DATA.DAT",0

READ 0,data2$

CLOSE 0

KILL F,"*"

OPEN "NEW.DAT",0

SEEK 0,0

WRITE 0,"Auto create a new file in FLASH."

SEEK 0,0

READ 0,data3$

CLOSE 0

SIZE 4,1

GAP 0,0

CLS

TEXT 10,10,"3",0,1,1,data1$

TEXT 10,60,"3",0,1,1,data2$

TEXT 10,110,"3",0,1,1,data3$

PRINT 1

EOP

TEST

[See Also]  	DOWNLOAD, EOP, READ, WRITE, SEEK, CLOSE

READ

[Description]   This command is used to read data from downloaded data file.

[Syntax]       READ file handle, variables

| Parameter | Description |
| --- | --- |
| File handle | 0 or 1. |
| variables | string, integer or float point variable |

[Example]     DOWNLOAD "DATA1",20,COMPUTER

2000

12

DOWNLOAD "DATA2",16,Mouse

900

93

DOWNLOAD "DEMO.BAS"

SIZE 3,1

GAP 0,0

DIRECTION 1

I=0

Y=100

OPEN "DATA1",0

OPEN "DATA2",1

SEEK 0,0

SEEK 1,0

:Start

CLS

TEXT 10,10,"3",0,1,1,"*****READ COMMAND TEST*****"

TEXT 10,50,"3",0,1,1,"OPEN-READ DATA"+STR$(I+1)

ITEM$=""

READ I,ITEM$,P,Q

TEXT 10,Y,"2",0,1,1,ITEM$+"$"+STR$(P)

BARCODE 10,Y+25,"39",40,1,0,2,4,"PRICE*"+STR$(Q)+"="+STR$(P*Q)

Y=Y+100

PRINT 1

Y=100

IF I<=1 THEN

IF EOF(I)=1 THEN

I=I+1

GOTO Start

ELSE

GOTO Start

ENDIF

ELSE

END

ENDIF

EOP

DEMO

[See Also]     DOWNLOAD, EOP, OPEN, EOF, LOF, SEEK, FREAD$()

SEEK

[Description]   This command is used to shift the specified file pointer to a certain position.

[Syntax]       SEEK file handle, offset

| Parameter | Description |
| --- | --- |
| File handle | 0 or 1. |
| offset | the offset characters which are shifted to the beginning of a new position |

[Example]     DOWNLOAD "DATA",12,1234567890

DOWNLOAD "TEST.BAS"

SIZE 4,1.5

GAP 0,0

DIRECTION 1

REFERENCE 0,0

CLS

OPEN "DATA",0

SEEK 0,4

READ 0,Num$

TEXT 100,10,"3",0,1,1,"SEEK COMMAND TEST"

BAR 100,40,300,4

TEXT 100,60,"3",0,1,1,"SHIFT 4 CHARACTERS"

TEXT 100,110,"3",0,1,1,Num$

BAR 100,140,300,4

SEEK 0,0

READ 0,Num$

TEXT 100,160,"3",0,1,1,"SHIFT 0 CHARACTERS"

TEXT 100,210,"3",0,1,1,Num$

PRINT 1

EOP

TEST

[See Also]     DOWNLOAD, EOP, OPEN, READ, EOF, LOF, FREAD$()

LOF( )

[Description]   This function returns the size of the specified file.

[Syntax]       LOF(“FILENAME”)

| Parameter | Description |
| --- | --- |
| FILENAME | The file downloaded in the printer memory. |

[Example]     DOWNLOAD "DATA1",10,1234567890

DOWNLOAD "DATA2",15,ABCDEFGHIJKLMNO

DOWNLOAD "LofTest.BAS"

SIZE 4,1.5

GAP 0,0

DIRECTION 1

CLS

OPEN "DATA1",0

OPEN "DATA2",1

TEXT 10,20,"4",0,1,1,"LOF() FUNCTION TEST"

J=LOF("DATA1")

K=LOF("DATA2")

TEXT 10,140,"3",0,1,1,"DATA1 IS: "+STR$(J)+" Bytes"

TEXT 10,200,"3",0,1,1,"DATA2 IS: "+STR$(K)+" Bytes"

PRINT 1

EOP

LofTestt

[See Also]     DOWNLOAD, EOP, OPEN, READ, EOF, SEEK, FREAD$()

FREAD$( )

[Description]   This function reads a specified number of bytes of data from a file.

[Syntax]       FREAD$ (file handle, byte)

| Parameter | Description |
| --- | --- |
| File handle | Either 0 or 1. |
| byte | Number of bytes to be read |

[Example]     DOWNLOAD "DATA1",10,1234567890

DOWNLOAD "DATA2",15,ABCDEFGHIJKLMNO

DOWNLOAD "OPEN2.BAS"

SIZE 4,1

GAP 0,0

DIRECTION 1

CLS

OPEN "DATA1",0

OPEN "DATA2",1

SEEK 0,0

SEEK 1,0

Y$=FREAD$(0,6)

Z$=FREAD$(1,6)

TEXT 10,100,"3",0,1,1,"FREAD$(0,6) IS: "+Y$

TEXT 10,150,"3",0,1,1,"FREAD$(1,6) IS: "+Z$

PRINT 1

EOP

OPEN2

[See Also]     DOWNLOAD, EOP, OPEN, READ, EOF, LOF(), SEEK

FOR…NEXT

[Description]  	Loop is used to execute one or more lines of program repetitively. A loop counter value specifies the number of executions. Nested loops are allowed (up to 39 nested loops) in this printer. Jumping out in the middle of the FOR…NEXT loop is prohibited. 、

[Syntax]       FOR variable = start TO end STEP increment

statement; start < end

[EXITFOR]

NEXT

| Parameter | Description |
| --- | --- |
| variable | The variable name is up to 8 characters |
| start | Integer or floating point numbers |
| end | Integer or floating point numbers |
| increment | nteger or floating point, positive or negative. |
| EXITFOR | Exit for loop |

[Example]     DOWNLOAD "TEST.BAS"

SIZE 4,2.5

GAP 0,0

CLS

FOR I=1 TO 10 STEP 1

TEXT 100,10+30*(I-1),"3",0,1,1,STR$(I)

NEXT

FOR I=1 TO 1000 STEP 100

TEXT 200,10+((I-1)/10)*3,"3",0,1,1,STR$(I)

NEXT

FOR I=110 TO 10 STEP -10

TEXT 300,10+(ABS(I-110))*3,"3",0,1,1,STR$(I)

NEXT

FOR I=1 TO 5 STEP 0.5

IF I-INT(I)=0 THEN Y=10+60*(I-1) ELSE Y=Y+30

TEXT 400,Y,"3",0,1,1,STR$(I)

NEXT

PRINT 1

EOP

TEST

[See Also]     DOWNLOAD, EOP

IF…THEN…ELSE…ENDIF

[Description]   Use IF…THEN block to execute one or more statements conditionally. Either a single-line syntax or multiple-line “block” syntax can be used:

[Syntax]       IF condition THEN statement

Notice that the single-line form of IF …THEN does not use an ENDIF statement.

Or

IF condition THEN

Statements

ELSE

Statements

ENDIF

Or

IF condition 1 THEN

Statement block 1

ELSEIF condition 2 THEN

Statement block 2

．．．

ELSEIF condition n THEN

Statement block n

ENDIF

The syntax of IF…THEN…ELSE requires that the command be typed in one single line in less than 255 characters.

| Parameter | Description |
| --- | --- |
| condition | Available relational operator: <, >, =, <=, >= |
| statement | Only one statement is available in |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 3,3

GAP 0.12,0

SPEED 4

DENSITY 8

DIRECTION 1

REFERENCE 0,0

OFFSET 0.00

SET PEELOFF

CLS

A=0

B=0

C=0

D=0

E=0

F=0

G=0

H=0

J=0

K=0

L=0

FORI=1 TO 100

IFI-INT(I/1)*1=0 THEN A=A+I

IFI-INT(I/2)*2=1 THEN B=B+I ELSE C=C+I

IFI-INT(I/3)*3=0 THEN

D=D+I

ENDIF

IFI-INT(I/5)*5=0 THEN

E=E+I

ELSE

F=F+I

ENDIF

IFI-INT(I/7)*7=0 THEN

G=G+I

ELSEIFI-INT(I/17)*17=0 THEN

H=H+I

ELSEIFI-INT(I/27)*27=0 THEN

J=J+I

ELSEIFI-INT(I/37)*37=0 THEN

K=K+I

ELSE

L=L+I

ENDIF

NEXT

TEXT 100,110,"3",0,1,1,"(1) 1+2+3+...+100="+STR$(A)

TEXT 100,160,"3",0,1,1,"(2) 1+3+5+...+99="+STR$(B)

TEXT 100,210,"3",0,1,1,"(3) 2+4+6+...+100="+STR$(C)

TEXT 100,260,"3",0,1,1,"(4) 3+6+9+...+99="+STR$(D)

TEXT 100,310,"3",0,1,1,"(5) 5+10+15+...+100="+STR$(E)

TEXT 100,360,"3",0,1,1,"  (1)-(5)="+STR$(F)

TEXT 100,410,"3",0,1,1,"(6) 7+14+21+...+98="+STR$(G)

TEXT 100,460,"3",0,1,1,"(7) 17+34+51+...+85="+STR$(H)

TEXT 100,510,"3",0,1,1,"(8) 27+54+...+81="+STR$(J)

TEXT 100,560,"3",0,1,1,"(9) 37+74="+STR$(K)

TEXT 100,610,"3",0,1,1,"  (1)-(6)-(7)-(8)-(9)="+STR$(L)

PRINT 1,1

EOP

DOWNLOAD “IFTHEN.BAS”

SIZE 3,4

GAP 0,0

DENSITY 8

SPEED 3

DIRECTION 0

REFERENCE 0,0

SET PEELOFF

CLS

A=50

B=5

C$=""

D$=""

:L1

IF A>100 THENGOTO L1 ELSE A=A+10

C$=STR$(A)+"IS SMALLER THAN 100"

TEXT 100,10,"4",0,1,1,C$

PRINT 1

END

:L2

A=A+B

D$=STR$(A)+"IS LARGER THAN 100"

TEXT 100,100,"4",0,1,1,D$

PRINT 1

GOTO L1

EOP

Note:

If the result of the expression is nonzero, the statement following THEN will be executed. If the result of the expression is zero, and the statement following the ELSE is present, it will be executed.Otherwise the next line of statement is executed.

If there are block of statements in IF…THEN …ELSE, ENDIF must be used at the end of the

IF…THEN …ELSE statement.

Limitations:

The total numbers of nested IF …THEN …ELSE statement in a program cannot exceed 40.

The total numbers of nested IF …THEN …ELSE, FOR…NEXT, GOSUB RETURN in a program cannot exceed 40 loops.

[See Also]     DOWNLOAD, EOP

GOSUB…RETURN

[Description]   This command will branch to a subroutine, executing statements until “RETURN” is reached.

[Syntax]       GOSUB LABEL

Statement

END

:LABEL

statement

RETURN

| Parameter | Description |
| --- | --- |
| LABEL | Beginning of the subroutine. The maximum length of the label is 8 characters. |

[Example]     DOWNLOAD "GOSUB1.BAS"

SIZE 4,3

GAP 0,0

DIRECTION 1

CLS

TEXT 10,10,"3",0,1,1,"GOSUB & RETURN COMMAND TEST"

GOSUB DR_BOX

PRINT 1

END

:DR_BOX

FOR I=21 TO 81 STEP 10

BOX 80+I,80+I,80+300-I,80+300-I,5

NEXT

RETURN

EOP

GOSUB1

[See Also]     DOWNLOAD, EOP, END, GOTO

GOTO

[Description]  	This command is used to branch to a specified label. The label can not exceed 8 characters in length.

[Syntax]       GOTO LABEL

:LABEL

| Parameter | Description |
| --- | --- |
| LABEL | Beginning of the point. The maximum length of the label is 8 characters. |

[Example]     DOWNLOAD "GOTO1.BAS"

SIZE 4,3

GAP 0,0

DIRECTION 1

CLS

A=0

TOTAL=0

:START

IF A<100 THEN

GOTO SUM

ELSE

GOTO PRTOUT

ENDIF

:SUM

A=A+1

TOTAL=TOTAL+A

GOTO START

:PRTOUT

B$="THE SUMMATION OF 1..100 IS

"+STR$(TOTAL)

TEXT 10,100,"3",0,1,1,B$

PRINT 1

END

EOP

[See Also]     DOWNLOAD, EOP, END, GOSUB…RETURN

REM

[Description]   Comment. Prefix is “REM” that will be ignored by the printer.

[Syntax]       REM

[Example]     REM ********************************

REM This is a demonstration program*

REM ********************************

DOWNLOAD "REMARK.BAS"

SIZE 4,3

GAP 0,0

DIRECTION 1

CLS

TEXT 50,50,"3",0,1,1,"REMARK DEMO PROGRAM"

REM TEXT 50,100,"3",0,1,1,"REMARK DEMO PROGRAM"

PRINT 1,1

EOP

REMARK

[See Also]     DOWNLOAD, EOP, END

INT( )

[Description]   This function is used to truncate a floating point number.

[Syntax]       INT (n)

| Parameter | Description |
| --- | --- |
| n | n can be positive or negative integer, floating point number or mathematical expression. |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 4,1

GAP 0,0

DIRECTION 1

INPUT "Number:",Num

CLS

REM **** To round up or down****

N=INT(Num+0.5)

IF N>Num THEN

TEXT 50,100,"3",0,1,1,"To round up= "+STR$(N)

ELSE

TEXT 50,100,"3",0,1,1,"To round down= "+STR$(N)

ENDIF

PRINT 1

EOP

56.2

[See Also]     DOWNLOAD, EOP, END, ABS(), ASC(), STR$()

LEFT$( )

[Description]   This function returns the specified number of characters down from the initial character of a string.

[Syntax]       LEFT$ (X$, n)

| Parameter | Description |
| --- | --- |
| X$ | The string to be processed |
| n | The number of characters to be returned |

[Example]     DOWNLOAD "TEST.BAS"

SIZE 4,1

GAP 0,0

DIRECTION 1

A$="BARCODE PRINTER DEMO PRINTING"

C$=LEFT$(A$,10)

CLS

TEXT 10,10,"3",0,1,1,A$

TEXT 10,100,"3",0,1,1,"10 LEFT 10 CHARS: "+C$

PRINT 1

EOP

TEST

[See Also]     DOWNLOAD, EOP, END, RIGHT$(), MID$(), LEN(), STR$()

LEN( )

[Description]   This function returns the length of a string.

[Syntax]       LEN (string)

| Parameter | Description |
| --- | --- |
| string | The string whose length is to be measured. |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 4,1

GAP 0,0

DIRECTION 1

A$="ABCDEFGHIJKLMNOPQRSTUVWXYZ"

B=LEN(A$)

CLS

TEXT 10,10,"3",0,1,1,A$

TEXT 10,50,"3",0,1,1,"STRING LENGTH="+STR$(B)

PRINT 1

EOP

DEMO

[See Also]     DOWNLOAD, EOP, END, LEFT$(), LEN(), RIGHT$(), MID$(), STR$(),

VAL()

MID$( )

[Description]  	This function is used to get the specified number of characters down from the mth character of a string.

[Syntax]       MID$(string, m, n)

| Parameter | Description |
| --- | --- |
| string | The string to be processed. |
| m | The beginning of mth characters in the string. 1 mstring length |
| n | The number of characters to return. |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 4,1

GAP 0,0

DIRECTION 1

A$="ABCDEFGHIJKLMNOPQRSTUVWXYZ"

E$=MID$(A$,11,10)

CLS

TEXT 10,10,"3",0,1,1,A$

TEXT 10,200,"3",0,1,1,"10 MIDDLE CHARS: "+E$

PRINT 1

EOP

DEMO

[See Also]     DOWNLOAD, EOP, END, LEFT$(), LEN(), RIGHT$(), STR$(), VAL()

RIGHT$( )

[Description]   This function returns the specified number of characters up from the end of a string.

[Syntax]       RIGHT$ (X$, n)

| Parameter | Description |
| --- | --- |
| X$ | The string to be processed |
| n | The number of characters to be returned from the right side (end) of the string. |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 4,1

GAP 0,0

DIRECTION 1

A$="ABCDEFGHIJKLMNOPQRSTUVWXYZ"

D$=RIGHT$(A$,10)

CLS

TEXT 10,10,"3",0,1,1,A$

TEXT 10,150,"3",0,1,1,"10 RIGHT CHARS: "+D$

PRINT 1

EOP

DEMO

[See Also]     DOWNLOAD, EOP, END, LEFT$(), LEN(), MID$(), STR$(), VAL()

STR$( )

[Description]   This function converts a specified value or expression into corresponding string of characters.

[Syntax]       STR$ (n)

| Parameter | Description |
| --- | --- |
| n | An integer, floating point number or mathematical expression. |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 4,1

GAP 0,0

DIRECTION 1

A$="ABCDEFGHIJKLMNOPQRSTUVWXYZ"

F=100

G=500

H$=STR$(F+G)

CLS

TEXT 10,10,"3",0,1,1,A$

TEXT 10,60,"3",0,1,1,"F="+STR$(F)

TEXT 10,110,"3",0,1,1,"G="+STR$(G)

TEXT 10,160,"3",0,1,1,"F+G="+H$

PRINT 1

EOP

DEMO

[See Also]     DOWNLOAD, EOP, END, LEFT$(), LEN(), RIGHT$(), MID$(), VAL()

VAL( )

[Description]   This function is used to convert numeric character into corresponding integer or floating point number.

[Syntax]       VAL(“numeric character”)

| Parameter | Description |
| --- | --- |
| numeric character | “0~9”, “.” |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 4,1

GAP 0,0

DIRECTION 1

A$="ABCDEFGHIJKLMNOPQRSTUVWXYZ"

F$="100"

G$="500"

CLS

H=VAL(F$)+VAL(G$)

I$=STR$(H)

TEXT 10,10,"3",0,1,1,A$

TEXT 10,60,"3",0,1,1,"F="+F$

TEXT 10,110,"3",0,1,1,"G="+G$

TEXT 10,160,"3",0,1,1,"F+G="+I$

PRINT 1

EOP

DEMO

[See Also]	DOWNLOAD, EOP, END, LEFT$(), LEN(), RIGHT$(), MID$(), STR$()

BEEP

[Description]   This command is used to issue a beep sound on portable keyboard.

[Syntax]       BEEP

| Parameter | Description |
| --- | --- |
| None | N/A |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 4,4

GAP 0,0

DIRECTION 1

BEEP

INPUT "Text1 =",TEXT1$

CLS

TEXT 100,100,"3",0,1,1,TEXT1$

PRINT 1

EOP

Device Reconfiguration Commands

SET COUNTER

[Description]  	Counter can be a real counter or a variable. This setting sets the counter number in program and their increments. There are three different kind of counters: digit (0~9~0), lower case letter (a~z~a) and upper case letter (A~Z~A).

[Syntax]       SET COUNTER @n step

@n = “Expression”

| Parameter | Description |
| --- | --- |
| @n | n: counter number. There are 61 counters available (@0 ~ @60) in the printer. @0 to @50 will be cleared while restarting the printer. @51 to @60 will be stored in printer until the printer is restored to factory default.@51~@55 were supported since V6.37 EZ.@56~@60 were supported since V6.74 EZ. |
| step | The increment of the counter, can be positive or negative.-999999999step999999999 If the counter is used as a fixed variable, please set the increment to 0. |
| Expression | Initial string. String length is 101 bytes |

[Example]     SET COUNTER @0+1

SET COUNTER @1+0

SET COUNTER @2-1

SET COUNTER @3 1

@0="0001"

@1="0101"

@2="000A"

@3="1"

SIZE 4,0.5

GAP 0,0

DIRECTION 1

CLS

TEXT 600,10,"3",0,1,1,3,"@0 @1 @2"

TEXT 600,30,"3",0,1,1,3,"Label "+@3+" --------------------------"

TEXT 600,50,"3",0,1,1,3,@0+" "+@1+" "+@2

PRINT 5

[See Also]     PRINT, TEXT, BARCODE

SET KEY1, SET KEY2

[Description]  	This setting is used to enable/disable the KEY1/KEY2function.The default function of KEY1 is “FEED” key, KEY2 is “PAUSE” key. Before setting KEY1/KEY2function otherwise, please disable KEY1/KEY2first. The setting will remain resident in the printer even when the printer is power off.

[Syntax]       SET KEY1ON/OFF

SET KEY2ON/OFF

SET KEY3 ON/OFF

| Parameter | Description |
| --- | --- |
| ON | Enable KEYn function |
| OFF | Disable KEYn function |

Note: The setting will remain in the printer even if the printer is power off.

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 3,1

GAP 0,0

DENSITY 8

SPEED 3

DIRECTION 0

REFERENCE 0,0

[See Also]     OFFEST, PRINT

SET PEEL

[Description]  	This setting is used to enable/disable the self-peeling function. The default setting for this function is off. When this function is set on, the printer stops after each label printing, and does not print the next label until the peeled label is taken away. This setting will be saved in printer memory when turning off the power.

[Syntax]       SET PEELON/OFF

| Parameter | Description |
| --- | --- |
| ON | Enable the self-peeling function |
| OFF | Disable the self-peeing function |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 3,1

GAP 0,0

DENSITY 8

SPEED 3

DIRECTION 0

REFERENCE 0,0

SET CUTTER OFF

SET KEY1 OFF

SET KEY2 OFF

SET KEY3 OFF

KEY1=0

KEY2=0

KEY3=0

:START

IF KEY1=1 THEN

CLS

TEXT 100,10,"3",0,1,1,"KEY1 (MENU key) is pressed!! "

PRINT 1,1

ELSEIF KEY2=1 THEN

CLS

TEXT 100,10,"3",0,1,1,"KEY2 (PAUSE key) is pressed!!"

PRINT 1,1

ELSEIF KEY3=1 THEN

CLS

TEXT 100,10,"3",0,1,1,"KEY3 (FEED key) is pressed!!"

TEXT 100,60,"3",0,1,1,"End of test"

PRINT 1,1

SET KEY1 ON

SET KEY2 ON

SET KEY3 ON

END

ENDIF

GOTO START

EOP

DEMO

[See Also]     OFFEST, PRINT

SET TEAR &SET STRIPPER

[Description]   This command is used to enable/disable feeding label to gap/black mark position for tearing off. This setting will be saved in printer memory when turning off the power.

[Syntax]       SET TEARON/OFF      	(TSPL2 language printers only)

SET STRIPER ON/OFF  (TSPL language printers only)

| Parameter | Description |
| --- | --- |
| ON | The label gap will stop at the tear off position after print. |
| OFF | The label gap will NOT stop at the tear off position after print. The beginning of label will be aligned to print head. |

[Example]     REM ***TEAR FUNCTION ON***

SIZE 3,3

GAP 0.08,0

DENSITY 8

SPEED 4

DIRECTION 0

REFERENCE 0,0

SET CUTTER OFF

SET PEEL OFF

SET TEAR ON

CLS

TEXT 50,100,"3",0,1,1,"TEAR FUNCTION TEST"

PRINT 1

[See Also]     SET PEEL, SET CUTTER

SET HEAD

[Description]   This setting is used to enable/disable head open sensor. If the head open sensor is turned off, an open printer head will not return an error message. This setting will be saved in printer memory. This command is only available for BPL2 printers.

[Syntax]       SET TEARON/OFF

| Parameter | Description |
| --- | --- |
| ON | Turn on the “HEADOPEN” sensor |
| OFF | Turn off the “HEADOPEN” sensor |

[Example]     SET HEADON

SET HEADOFF

SET RIBBON

[Description]   This setting is used to enable/disable ribbon sensor detection. (Thermal Transfer Printing/Thermal Direct Printing) Printer will detect the presence of a ribbon to determine using either direct thermal or thermal transfer printing upon printer startup. This setting will NOT be saved in printer memory.

[Syntax]       SET RIBBON ON/OFF/INSIDE/OUTSIDE

| Parameter | Description |
| --- | --- |
| ON | Thermal transfer printing |
| OFF | Thermal direct printing |
| INSIDE | The ribbon is inside wound. For TTP-384M only. *Since V6.80EZ. |
| OUTSIDE | The ribbon is outside wound. For TTP-384M only. *Since V6.80EZ. |

[Example]     REM *****Disable ribbon detection sensor for direct thermal printing.

SET RIBBON OFF

SIZE 4,1

GAP 0,0

CLS

TEXT 10,10,"3",0,1,1,"Direct thermal printing."

PRINT 1

REM *****Enable ribbon detection sensor for thermal transfer printing.

SET RIBBON ON

SIZE 4,1

GAP 0,0

CLS

TEXT 10,10,"3",0,1,1,"Thermal transfer printing."

PRINT 1

REM *****For using ink-in ribbon in TTP-384M.

SET RIBBON INSIDE

SIZE 4,1

GAP 0,0

CLS

TEXT 10,10,"3",0,1,1,"TTP-384M is using ink-in ribbon."

PRINT 1

REM *****For using ink-out ribbon in TTP-384M.

SET RIBBON OUTSIDE

SIZE 4,1

GAP 0,0

CLS

TEXT 10,10,"3",0,1,1,"TTP-384M is using ink-out ribbon."

PRINT 1

SET COM1

[Description]   This setting defines communication parameters for printer serial port.

[Syntax]       SET COM1 baud, parity, data, stop

| Parameter | Description |
| --- | --- |
| baud | Baud rate, available baud rates are as listed : 24: 2400 bps 48: 4800 bps 96: 9600 bps 19: 19200 bps 38: 38400 bps 57: 57600 bps 115: 115200 bps |
| Parity | Parity check N: None parity check E: Even parity check O: Odd parity check |
| data | Data bit 8: 8 bits data 7: 7 bits data |
| stop | Stop bit 1: 1 stop bit 2: 2 stop bits |

[Example]    The parallel port is used to setup the printer serial port in this example via MS-DOS mode.

C:\>COPY CON LPT1<ENTER>

SET COM1 19,N,8,1<ENTER>

<CTRL><Z><ENTER>

C:\>

Note:

<ENTER> stands for PC keyboard “ENTER” key.

<CTRL><Z> means to hold PC keyboard “CTRL” key then press

PC keyboard <Z> key.

SETPRINTKEY

[Description]  	This command will print one label and feed label gap to tear bar position for tearing away. Press FEED button to print the next label or batch of labels. If label content includes serial text or barcode, it will change the serial number accordingly. This setting will be saved in printer memory.

[Syntax]       SET PRINTEKYOFF/ON/AUTO/<num>

| Parameter | Description |
| --- | --- |
| OFF | Disable this function |
| ON | Enable this function |
| AUTO | Enable this function |
| <num> | Numbers of labels will be printed if FEED button is pressed. |

[Example]     SIZE 4,2.5

GAP 0.12,0

SET PRINTKEY ON

SET COUNTER @0 1

@0="0001"

CLS

TEXT 10,10,"5",0,1,1,@0

PRINT 1

Execute:

| Syntax | Receive “PRINTm” | Print Out |
| --- | --- | --- |
| SET PRINTKEYON or SET PRINTKEY AUTO | 1.) PRINT 2 | Label 1~2 |
|  | 2.) Press FEED key | Label 3~4 |

| Syntax | Receive “PRINT m,n” | Print Out |
| --- | --- | --- |
| SET PRINTKEYON or SET PRINTKEY AUTO | 1.) PRINT 1,2 | Label 1, Label 1 |
|  | 2.) Press FEED key | Label 2, Label 2 |

| Syntax | Receive “PRINT -1,n” | Print Out |
| --- | --- | --- |
| SET PRINTKEYON or SET PRINTKEY AUTO | 1.) PRINT -1,2 | Label 1, Label 1 |
|  | 2.) Press FEED key | Label 1, Label 1 |

| Syntax | Receive “PRINTm” | Print Out |
| --- | --- | --- |
| SET PRINTKEY 5 | 1.) PRINT 2 | Label 1~2 |
|  | 2.) Press FEED key | Label 3~7 |

| Syntax | Receive “PRINT m,n” | Print Out |
| --- | --- | --- |
| SET PRINTKEY 5 | 1.) PRINT 1,2 | Label 1, Label 1 |
|  | 2.) Press FEED key | Label 2~6 |

| Syntax | Receive “PRINT -1,n” | Print Out |
| --- | --- | --- |
| SET PRINTKEY 5 | 1.) PRINT -1,2 | Label 1, Label 1 |
|  | 2.) Press FEED key | Label 1, Label 1 |

SET REPRINT

[Description]   This command will disable/enable a reprinting attempt subsequent to a "no paper", "no ribbon" or "carriage open" error.

[Syntax]       SET REPRINTOFF/ON

| Parameter | Description |
| --- | --- |
| OFF | Disable this function |
| ON | Enable this function |

[Example]     SET REPRINTON

PEEL

[Description]   This command is used to obtain status of the peel-off sensor. Its attribute is read only.

[Syntax]       PEEL

| Parameter | Description |
| --- | --- |
| 0 | Paper is not on top of peel sensor |
| 1 | Paper is on top of peel sensor |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 4,1

GAP 0,0

SET PEEL OFF

SET KEY1 OFF

SET LED1 OFF

SET LED3 OFF

:START

LED1=0

LED3=0

IF KEY1=1 THEN GOTO A

GOTO START

:A

LED1=1

CLS

TEXT 10,10,"3",0,1,1,"PEEL Function Test!! "

PRINT 1,1

:B

LED1=0

IF PEEL=1 THEN

LED3=1

GOTO B

ELSE

CLS

TEXT 10,10,"3",0,1,1,"The label is removed from the PEEL sensor!!"

PRINT 1,1

GOTO START

ENDIF

EOP

DEMO

KEY1, KEY2

[Description]   This command is used to read the status of KEY1,KEY2 .

[Syntax]       KEY m=n

| Key | Return Value |
| --- | --- |
| KEY1 (FEED) | 0: released 1: pressed |
| KEY2(PAUSE) | 0: released 1: pressed |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 3,1

GAP 0,0

SPEED 4

DENSITY 8

DIRECTION 1

REFERENCE 0,0

SET KEY1OFF

:START

IF KEY1=1 THEN

CLS

TEXT 100,10,"3",0,1,1,"KEY FUNCTION TEST"

PRINT 1,1

GOTO START

EOP

DEMO

Printer Global Variables

@LABEL

[Description]   This variable is used to count how many pieces of labels have been printed. It can’t be initialized if the printer is reset. It will be memorized if the printer power is turned off.

[Syntax]       Write attribute: @LABEL=n or @LABEL=”n”

Read attribute: A=LABEL or A$=STR$(LABEL)

| Parameter | Description |
| --- | --- |
| n | Number of labels printed. 0n999999999 |

[Example]     DOWNLOAD "DEMO.BAS"

SIZE 4,2.5

GAP 0,0

DIRECTION 1

CLS

TEXT 10,50,"3",0,1,1,@LABEL

TEXT 10,100,"3",0,1,1,"@LABEL="+STR$(LABEL)

TEXT 10,150,"3",0,1,1,"*****Statement 1*****"

IF LABEL>1000 THEN

TEXT 10,200,"3",0,1,1,"LABEL>1000"

ELSE

TEXT 10,200,"3",0,1,1,"LABEL<1000"

ENDIF

TEXT 10,250,"3",0,1,1,"*****Statement 1*****"

A=LABEL

IF A>1000 THEN

TEXT 10,300,"3",0,1,1,"A>1000"

ELSE

TEXT 10,300,"3",0,1,1,"A<1000"

ENDIF

TEXT 10,350,"3",0,1,1,"*****Statement 3*****"

A$=STR$(LABEL)

IF VAL(A$)>1000 THEN

TEXT 10,400,"3",0,1,1,"VAL(A$)>1000"

ELSE

TEXT 10,400,"3",0,1,1,"VAL(A$)<1000"

ENDIF

PRINT 1,1

EOP

DEMO